RPLIDAR A2M12

This document explains how to build the RPLIDAR SDK and run the ultra_simple example on a Linux system.

PFDT

# RPLIDAR SDK – Linux Build & Run Guide

## 1. System Requirements

- Linux (Ubuntu 20.04/22.04/24.04 recommended)

- CMake (≥ 3.5)

- GCC / G++

- RPLIDAR connected via USB

## 2. Install Dependencies

sudo apt update
sudo apt install build-essential cmake

## 3. Clone or Extract SDK

If using Git:

cd ~/Music
git clone https://github.com/Slamtec/rplidar_sdk.git
cd rplidar_sdk

Or extract the downloaded ZIP and navigate into the folder:

cd ~/Music/rplidar_sdk_master

## 4. Build the SDK

mkdir build
cd build
cmake ..
make

After successful compilation, executables will be generated.

## 5. Navigate to Example Binary

cd output/Linux/Release

(If you built inside /build, adjust path accordingly.)

## 6. Verify Serial Port

Check where the device is connected:

ls /dev/ttyUSB*

Typical output:

/dev/ttyUSB0

## 7. Run ultra_simple Example

./ultra_simple --channel --serial /dev/ttyUSB0 256000

## 8. Parameter Explanation

| Argument | Description |
| --- | --- |
| --channel --serial | Use serial communication mode |
| /dev/ttyUSB0 | Serial port of the LiDAR |
| 256000 | Baud rate (common for A1/A2 models) |

## 9. Permission Configuration (If Required)

If you get a Permission denied error:

sudo usermod -aG dialout $USER

Then log out and log in again.

## 10. Expected Output

If the device is functioning correctly, LiDAR scan data will continuously print in the terminal.

### Notes

- Ensure the LiDAR motor starts spinning when the program runs.

- Baud rate may differ depending on the RPLIDAR model.

- Confirm the correct serial port before execution.